Documentation for TL.EXE Program

Ver. 3.09, Jan 24, 1996

by R. Dean Straw, N6BV

Senior Assistant Technical Editor, ARRL

OVERVIEW

[Note: Version 3.09 fixed a silly little bug computing the

voltage maximum point along a mismatched transmission line.

Version 3.08 added the ability to change the unloaded Qc of the

capacitors in the tuning networks. Beyond Version 3.06, the

voltage ratings for some cables were revised per Belden's latest

catalog.]

"TL" is short for "Transmission Line." The TL program has been

under development, intermittently, for about 7 years. It started

out as a small, personal utility to analyze my own antenna and

feed-line system. Over the years, friends and associates asked

for new features; I also added a number of capabilities on my

own. Gradually, TL grew larger and larger, so that now there are

more than 2400 lines of code in the program. TL also got

complicated enough that I decided I had better add a

documentation file to go along it!

TL has a lot of capabilities. The original purpose was simply

to determine the impedance at the shack-end of a transmission

line terminated with a complex load impedance. Modern Method-of-

Moment programs like NEC can accurately compute the feed-point

impedance of an antenna over actual ground. Essentially, TL

allowed me to continue the antenna analysis down into the shack.

TL uses the hyperbolic Transmission Line Equation to compute

the line input impedance. To do that, it needs the type of line

(coax RG type or open-wire type), the physical length of the

line, and the frequency of operation. TL also determines the

losses on the line, separated into two components: the inherent

matched-line loss, and the additional loss due to SWR when the

load is not equal to Z0 of the line.

About four years ago, I added the ability to picture various

antenna tuner configurations that could be used to transform the

impedance at the input end of a transmission line to 50 ohms.

This was for "perfect" components without any losses.

About two years after that addition, I worked extensively with

Don Patterson, KK6JI. I thus became curious about the sort of

stresses the line is subject to under various load conditions. So

I added a search for the maximum RMS voltage along the line, in 1

degree increments. (For your information, the highest voltage

will usually be near the load end of a line terminated in a

highly reactive load impedance.)

TL stabilized for awhile in this configuration, until Frank

Witt, AI1H, started investigating the losses in various antenna

tuner configurations. Frank had developed an elegant approach to

1

measuring real tuners using simple instrumentation. While Frank

had used TL to rapidly evaluate numerous test cases, he wanted

more information. He persuaded me to expand the program (to about

double its original size) to add iterative search algorithms. TL

could thus determine explicitly the losses in the various

components making up real antenna tuners.

CAVEATS

I must caution you at this point. TL displays results out to

two, or even three, decimal places. Internally, computations are

carried out to even more decimal places, of course. In the real

world, the one factor that varies the most in actual transmission

lines is the Velocity Factor. This may easily vary plus or minus

10% for typical lines -- in fact, the velocity factor may even

vary slightly for two pieces of cable cut from the same bulk

roll! Along with the Velocity Factor, the exact value for the

characteristic impedance Z0 also varies.

TL will give you a good indication of what you can expect in

the real world, but only plus or minus the velocity factor and

the actual impedance at the antenna feed point! Please remember:

TL is fundamentally an educational tool. It can also be used very

effectively as a design tool, provided that you know the exact

parameters of your transmission lines and your antennas. If TL

helps open your eyes about transmission lines and antenna tuners,

particularly the losses associated with each, then I will have

achieved my goal in writing it.

TL will complete a computation in a fraction of a second on a

powerful modern microcomputer, like the 80486DX/33MHz machine I

am using now to write this documentation file. It takes about

five seconds to work on an ancient 8088-based 4.77 MHz PC, with

an 8087 numeric coprocessor installed. Roughly the same amount of

time is needed for a 33 MHz 80486SX computer (without numeric

coprocessor.) TL employs a lot of heavy-duty math, so a numeric

coprocessor is very desirable.

The program is entirely character-based for output display.

Hence any IBM-PC compatible computer will work with TL. Hitting

[Shift] [Print Screen] will print out a TL screen on any printer

that recognizes the 8-bit IBM character set.

USING TL

TL is menu-driven and is reasonably "friendly." However, I

must assume that the user has some technical knowledge about

transmission lines and antenna tuners. The user must be familiar

with the so-called rectangular representation of complex

impedance, in the form Za = Ra +/- j Xa. Later in this file there

is a table of typical impedance data for several types of

antennas. You can use this data with TL to model realistic

situations.

You boot up TL once you have gotten into the subdirectory

2

containing TL.EXE, the executable file, using the CD [change

directory] command in DOS. Once there, Type:

TL [Enter]

The opening screen shows a menu of the various types of

transmission lines TL models. The first eight choices are

flexible coaxial cables, with "RG" designations. Choices 9

through 12 are Hardline coaxial cables. Choices 13 and 15 are for

two-wire balanced transmission lines, such as 300-ohm transmit-

ting line, 450-ohm "window ladder line" or 600-ohm wire line.

Choice number 16 is for "other" transmission lines not found

on the main menu. For this choice, the user manually enters the

characteristic impedance (both resistive and reactive parts), the

matched-line loss (in dB/100 feet), the velocity factor and the

maximum rms voltage for which the line is rated by its

manufacturer.

Choices 1 through 15 use the parameters listed in Chapter 24

of The ARRL Antenna Book, including the values for matched-line

attenuation versus frequency, found in Fig 22, on page 24-16.

(Note that the matched-line loss for 450-ohm "window" ladder line

has been revised to have the same slope as #12 open-wire line,

from the second printing of the 17th Edition. TL reflects that

change.)

If you merely hit the [Enter] key, TL will select the default

value of "4," meaning RG-8A/RG-213, 50-ohm cable solid-dielectric

cable. In most data entry points in TL, there is a default value,

indicated by square brackets; e.g., [4] in the main menu.

TL will then prompt you for the length of the transmission

line, in feet. The default value is zero feet -- this is useful

when evaluating antenna tuners by themselves, without an

intervening transmission line between the tuner and the load.

Just hit [Enter].

Next, you will be prompted for the operating frequency, in

MHz. The default is 3.5 MHz, simply by hitting the [Enter] key

without entering anything else. You can enter any frequency as

high as 5000 MHz (5 GHz), or as low as 0.002 MHz (20 kHz). After

you hit [Enter], TL will compute and display the matched-line

loss for the chosen line. The matched-line loss is for a load

equal to the characteristic impedance of the particular

transmission line chosen, for the length of line and the

frequency chosen.

Next, TL will prompt you to enter the resistive part of the

load impedance, in ohms. If you don't enter a number, but simply

hit the [Enter] key, TL will automatically enter a resistive

value of 0.00001 ohms. It doesn't enter zero ohms, because that

would result in embarrassing "divide by zero" problems later on.

After you hit [Enter] for the resistive part of the load

3

impedance, you will then be prompted to enter the reactive part

of the load, again in ohms. Note that a capacitive reactance must

be preceded with a "-" (minus) sign. An inductive reactance need

not be preceded with a "+" (plus) sign, although you may enter

one if you wish. Merely hitting [Enter] will enter a reactance of

zero ohms.

After you finish specifying the impedance, TL will do its

computations. It will present you with a screen showing the

information you entered, plus the SWR at the load, followed by

the SWR at the input of the transmission line. In general, the

two SWRs will be different. If the line is very lossy or the SWR

at the load is very high, the difference between the two SWR

values may be great, with the lower value at the input of the

line. Measuring the SWR at the input of a lossy line with a high

SWR at the load will mask the magnitude of the SWR at the end of

that line, and may possibly lull you into complacency as you

measure the SWR in the shack.

The next line on-screen shows the additional loss in the line

due to the SWR at the load, followed by a line showing the sum of

the matched-line loss and the additional loss due to SWR. This is

the total loss in the transmission line. The power computations

are based on a power of 1500 W at the input of the transmission

line. (I tacitly assume at this point that there is a lossless

antenna tuner to match the line's input impedance to the 50 ohms

needed by a typical transmitter.)

TL also displays the transformed impedance at the input of the

transmission line, both in rectangular (R +/- jX ohms) and polar

coordinates (magnitude in ohms, phase angle in degrees). For

1,500 W of rf into the input of the line, the maximum RMS voltage

along the line is displayed, along with the distance from the

load where the peak voltage occurs. Note that transmission lines

are rated by their manufacturers in terms of RMS voltage. TL

displays the rms voltage rating for the particular transmission

line chosen, for comparison. At the bottom of the screen is a

prompt to choose the next action -- the default is [T], for

antenna Tuner.

EVALUATING ANTENNA TUNER CONFIGURATIONS

If you select either "T" or [Enter], TL will erase the screen

and then display the Antenna Tuners menu. You can select one of

four different configurations:

1 = Low-Pass L-Network

2 = High-Pass L-Network

3 = Pi-Network

4 = Tee-Network

You can also exit from this menu back to either the main menu

or back to DOS if you like, by selecting "M" or "X" respectively.

Choose one of the antenna-tuner configurations. If you choose

either the pi-network or tee-network antenna tuners, you will be

4

prompted to enter the value, in pF, of the output capacitor in

the network. For the pi-network the default value is 500 pF, and

for the tee-network the default value is 100 pF.

Once you have entered the necessary information, TL will

compute the values needed to transform the impedance at the

antenna tuner output to 50 ohms. If the chosen network

configuration cannot perform the desired transformation, an

audible alarm will sound, and TL will recommend another network

or another output capacitor value to try.

The inductor is usually, but not always, the most lossy

component in an antenna tuner. TL assumes a default value for

unloaded Q of 200 for an inductor, a typical value for a

practical inductor mounted in a metal case.

The model for a lossy inductor is an inductive reactance in

series with a loss resistance. For example, if the unloaded Q is

200 and the inductive reactance at the chosen frequency is +400

ohms, then the loss resistance is 2 ohms in series with the +400

ohms reactance.

TL assumes that the unloaded Q for any capacitor in the

antenna tuner is 1000. Again, the model for a lossy capacitor is

the capacitive reactance in series with a small loss resistance.

The default value of unloaded Q = 1000 for a capacitor is typical

of transmitting variable capacitors.

The operator can modify the unloaded Q value for both the

inductors and capacitors in a tuner, using the "Q" prompt from

one of the screens showing a schematic antenna-tuner

representation. The unloaded Q values chosen will remain for

subsequent network computations.

THE ANTENNA TUNER SCREEN

Examine the antenna tuner screen carefully -- a lot of

information is displayed there! At the top of the display, there

is a summary of the transmission line parameters chosen: the

frequency, the type of line, and the length of the line. On the

next line the impedance at the input of the line is displayed, in

rectangular and polar forms. This is the impedance presented to

the antenna tuner to be transformed to 50 ohms.

Then TL shows something called the "Highest effective network

Q," also known commonly as the "loaded Q" of a network. This is

the highest Q in the network, at the specified load impedance,

and is an indication of how "touchy" the tuning will be. The

higher the effective network Q, the more carefully you must tune

the variable capacitor(s) and/or variable inductor in the tuner

in order to achieve the desired transformation to 50 ohms.

The loss in an antenna tuner is also closely related to the

effective network Q -- the higher the effective network Q, the

higher will be the loss. Efficiency in an L-C network is defined

5

as:

Efficiency (%) = 100 x (1 - (QL/QU))

where QL is the loaded Q, and QU is the unloaded Q of the

network components. See page 13.7 of the 1995 ARRL Handbook for

more details on this subject.

For a given network loaded Q ("effective network Q" in TL),

components with higher unloaded Qs will result in lower tuner

losses. This makes intuitive sense, especially if you recall that

"Q" stands for "Quality Factor," and higher unloaded Qs mean

higher quality, less lossy components.

TUNER LOSSES, DETAILS

Examine the line showing the estimated power loss in the

tuner. The loss shown is for an rf input to the tuner of 1,500 W,

the full Amateur legal limit. The loss is expressed in watts, in

dB, and also as a percentage of the 1,500 W at the input. For

example, a particular tuner configuration might lose 114 W out of

the 1,500 W put into it, yielding a loss of 0.34 dB, or 7.44% of

the input power. If the input to the tuner is 100 W, rather than

1,500 W, the loss would still be 0.34 dB, or 7.44% of 100 W =

7.44 W.

The next line on-screen summarizes the amount of loss in the

transmission line itself, plus the total loss in the line and the

antenna tuner, both expressed in dB.

STRESSING THE TUNER

Now examine carefully the table for the individual components

in the antenna tuner. The reactances for each element are shown

first, followed by the peak voltage, the RMS current, and the

power lost. Each is computed for 1,500 W of power. For lower

power levels, multiply the voltages and currents by the square

root of the ratio of power in use divided by 1500. For example,

if the power into the tuner is 100 W:

Multiplying factor = Square Root (100/1500) = 0.2582

Note that the voltage shown by TL is the peak voltage across

each component. Pardon the pun, but this is potentially a little

confusing, especially where a series element is concerned. What

is shown is not the voltage from the element to "ground" (the

common terminal); it is the voltage across the component itself.

In addition, the current shown is that flowing through each

component.

Look carefully at the on-screen schematic. Note that the

impedance actually seen at the output terminals of the tuner is

listed towards the top of the screen, not on the schematic

itself, due to lack of space. The load impedance at the end of

the transmission line is shown schematically, preceded by

6

squiggly lines that symbolize the intervening transmission line.

Exceeding the peak voltage rating across any component in a

tuner will probably cause an arc. Exceeding the current-carrying

ability for a component will often result in smoke, due to the

excessive amount of power dissipated in that element! The

inductor in a tuner will sometimes melt because of excess power

dissipation.

This occurs most frequently with low-resistive loads, with or

without a high reactive component. For example, you can simulate

a stressful situation by specifying a load impedance of 3 + j0 at

3.5 MHz, for a tee-network configuration having a 100 pF output

capacitor. The shunt inductor will not only have to withstand

more than 6000 V peak, but worse yet, it will have to dissipate

almost 700 W of power at more than 20 A of circulating current!

Toasted coil, if it doesn't arc first.

TL allows you to play around with various impedances, unloaded

Qs and different network configurations, without having to endure

the smoke and arcing that occurs in many tuners, even ones

supposedly rated to handle a "full gallon" of rf. Now, for fun,

increase the size of the output capacitor in the tee-network

and/or increase the unloaded Q of the inductor or the capacitors

to help unstress the beleaguered antenna tuner in the example

above -- or change to a lower-loss configuration than the tee-

network.

Now, let's try something really dramatic. At a frequency of

3.5 MHZ, enter a value of 0.00001 ohms for the resistive part of

the load, with a reactance of zero ohms. Then select the Tee

network, with an output series capacitor of 100 pF. The tuner

will absorb all 1500 W of input power -- in other words it tunes

up into itself with a short at the output!

In general, L-networks will exhibit the least loss among the

various network configurations, but they often require awkward

values for inductance and capacitance. The tee-network configura-

tion is often used because it can accommodate a wide range of

impedances with practical values of variable capacitors and

inductors, albeit with sometimes disastrous internal losses. The

pi-network configuration is flexible, but it too will often

require very large values for the shunt capacitors.

Another hint: from the screen showing the SWRs and losses in

the transmission line, you may bypass the antenna tuner

configuration menu by choosing directly "1", "2", "3" or "4",

corresponding to the number of the desired antenna tuner network,

followed by [Enter]. TL will then prompt you for the next piece

of information needed.

7

SAMPLE TEST DATA

100-foot long, center-fed dipole, 50 feet over ground with

dielectric constant (relative permittivity) of 13, conductivity

of 5 mS/m. Computed by NEC2 for flat-top configuration.

Freq.          Feedpoint

MHz            Impedance

----------------------------------

1.83 MHz       4.5  - j 1673 ohms

3.8 MHz         39  - j  362 ohms

7.1 MHz        481  + j  964 ohms

10.1 MHz      2584  - j 3292 ohms

14.1 MHz        85  - j  123 ohms

18.1 MHz      2097  + j 1552 ohms

21.1 MHz       345  - j 1073 ohms

24.9 MHz       202  + j  367 ohms

28.4 MHz      2493  - j 1375 ohms

66-foot long, center-fed inverted-V dipole, apex at 50 feet high

over ground with dielectric constant of 13, conductivity of 5

mS/m.

Freq.          Feedpoint

MHz            Impedance

-----------------------------------

1.83 MHz        1.6  - j 2257 ohms

3.8  MHz       10.3  - j  879 ohms

7.1  MHz       64.8  - j 40.6 ohms

10.1 MHz       21.6  + j  648 ohms

14.1 MHz       5287  - j 1310 ohms

18.1 MHz        198  - j  820 ohms

21.1 MHz        103  - j  181 ohms

24.9 MHz        269  + j  570 ohms

28.4 MHz       3089  + j  774 ohms

FEEDBACK, PLEASE

This is where TL stands. In a very complex program like this,

I'm sure people will find bugs. I'd really appreciate detailed

feedback concerning any problems found. My e-mail address at ARRL

HQ is rdstraw@arrl.org or by CompuServe: 73063,773.

8

####################################################################################################################