WeatherMan:

A Radiofacsimile Reception Program

for

IBM PCs and Compatibles

Table of Contents

(Find topics by searching for the topic title.  Be sure to include the

brackets in the search string.)

[Notice]

[I]...........What Is Wefax?

[II]..........How WeatherMan Works

[III].........Connecting a Radio to a Sound Card

[IV]..........Creating a Configuration File

[V]...........Running WeatherMan

[VI]..........WeatherMan Keyboard Commands

[VII].........The SHOWPIC Program

[VIII]........The RAW2PCX Program

[IX]..........WXDEMO.EXE

[Appendix 1]..Weather Facsimile Frequencies

[Appendix 2]..Troubleshooting Q&A

[Appendix 3]..Supported Sound Cards

[Appendix 4]..About the Sample Pictures

[Notice]

*****

Notice, Part 1-- The Plea for Cold, Hard Cash:

The computer programs and documentation described in this document are

neither free, nor are they in the public domain: they are shareware.

In this particular case, the term "shareware" means that you are welcomed

and encouraged to use the programs for a trial period of not more than 30

days.  If, after or during the trial period, you decide to continue

using the programs, you should send a check (drawn on a U.S. bank) or

a U.S. Postal Money Order in the amount of $20.00 (US currency) to the

program's author:

Paul Hitchcock

2309 Blake Street, #308

Berkeley, CA 94704

Email about the program may be sent to me at

Internet   : paulhtc@netcom.com

Compuserve : 72673,3522

*****

Please note that this program, unlike many other shareware offerings, is

*not* "cripple-ware"-- it is fully functional.  While it's true that

offering a limited version of a program via shareware might be an incentive

for users to register, I've always disliked the practice.  However,

registered users will go to the "head of the line" as far as support is

concerned.

Notice, Part 2-- The Ugly Legal Stuff:

The programs and documentation in this package are distributed "as-is",

with no warranty expressed or implied, including, but not limited to,

merchantability or fitness for a particular purpose.  Any and all

damages arising out of the use of the programs and/or documentation

are the responsibility of the purchaser and/or user of the programs

and documentation.

All programs and documentation in this package are Copyright (C) 1994

by Paul Hitchcock.

"Sound Blaster", "Sound Blaster Pro", and "Sound Blaster 16" are

registered trademarks of Creative Labs, Inc.

"PAS 16" and "ProAudio Spectrum" are registered trademarks of

Mediavision, Inc.

*****

[Introduction]

WeatherMan is a computer program for IBM PCs and compatibles which,

when used with a Sound Blaster-compatible sound card, lets you receive,

display, and store to disk weather photos and charts of the type

broadcast over the high-frequency (HF) shortwave bands.

To use the program you must have the following items:

1) A shortwave receiver, equipped with a beat-frequency

oscillator (BFO)

2) A 100% Sound Blaster-compatible sound card

3) A VGA video card and monitor capable of displaying

640x480 16-color graphics

4) An IBM PC or compatible 386/486 computer, running at

33 MHz or faster

5) A patch cord or microphone for connecting the shortwave

receiver to the sound card

[I]  What Is Wefax?

Wefax, or weather facsimile, is a technology for transmitting

photographs and other graphical material via radio.  The process

is fairly straightforward: At the transmitter, an object whose

image is to be transmitted is scanned line-by-line at a constant

rate by an optical sensor; the output of the sensor is used to

control the frequency of an oscillator in such a way that the

oscillator's output is linearly proportional to the amount of light

refected from corresponding areas of the scanned object.  The

frequency of the oscillator ranges from 1500 Hz for pure-black areas

to 2300 Hz for white regions.  This "frequency-modulated subcarrier"

is in turn used to frequency-modulate a radio wave which is then

broadcast.

At the receiver the process is reversed: the wefax signal is

demodulated, resulting in a set of tones whose frequencies

are again linearly proportional to the light and dark areas of the

original scanned object.  If an FM receiver is employed to detect

the signal, the original frequencies are faithfully reproduced.

More commonly, however, the signal is demodulated using an AM

receiver equipped with a beat-frequency oscillator (BFO).  While

FM reception generally results in better noise rejection, the BFO

method gives the listener the ability to choose (within limits,

of course) the center frequency of the 800 Hz audio band; this

capability can be very useful when trying to receive a fax in

the presence of an interfering signal.  BFO-based reception is

assumed throughout the remainder of the discussion.

For the purpose of this discussion, a wefax broadcast can be

defined by two parameters:  (1) its transmission rate (or "drum

speed")  in lines per minute (lpm) and (2) its index of correlation

(IOC).  The IOC is essentially the ratio of the transmitted

picture's line width to its line height.  At the present time,

nearly all U.S. (and many foreign) wefax broadcasts are transmitted

with a drum speed of 120 lpm and an IOC of 576. There are a number

of other radiofacsimile formats, but the present version of WeatherMan

can only receive faxes broadcast at 120 lpm.

A more complete description of weather facsimile broadcasting can

be found in the 1990 edition of the ARRL Handbook.

(Note: The term "wefax" is not really appropriate for describing

HF weather facsimile broadcasts, but since "everybody" uses it in

this context, I'll follow the same convention.)

[II] How WeatherMan Works

The digital sound processor (DSP) chip on sound cards such as the

Sound Blaster provides a number of sound-processing services.

One of these services, DMA (Direct Memory Access) audio recording,

may be integrated into a computer program, giving the program the

ability to record audio signals as a background task.  WeatherMan

uses this facility to insert a line of radiofax information into a

memory buffer at the same time it is processing and displaying the

previously received line.

Once the radiofax data is in the computer, WeatherMan determines the

frequency of the audio information by measuring the time interval

between consecutive zero-point crossings of the recorded samples.

Once the frequency has been determined, the program assigns a

proportional color number from 0 through 15 to the frequency, plots

the point, then goes back for more samples.  This process continues

until the program's user takes some intervening action.

In it's current incarnation, WeatherMan samples the mic input at a

rate of 10,000 samples per second and uses, on average, 8 samples

per pixel.  From these numbers, one would assume this would yield a

picture 625 pixels wide (5000 samples per half-second line divided

by 8).   Unfortunately, it's not quite that simple, for two principal

reasons:  1) Individual sound card/computer combinations have varying

sampling rates for which allowance must be made, and 2) setting up

and exiting the DMA record procedure requires processor time.  Since

the incoming radiofax transmission does not wait for WeatherMan to

complete its internal housekeeping before sending more data, a certain

amount of information is necessarily going to be lost from each received

line.  How much data is lost?  On average, the amount lost translates to

about two pixels per line.

First, the good news-- although two pixels per line are not plotted,

if the picture being received is properly aligned, no useful information

is lost.  The nature of fax transmission requires that data can't

be transmitted over the entire line: for example, a mechanical fax

receiver must be allowed a certain amount of time for the pen or plotting

stylus to return to the margin to begin drawing the next line.

Next, the bad news-- compromises had to be made.  In order for

WeatherMan to run on the greatest number of machines, I decided

that the screen resolution should be no greater than 640x480 in

16 colors.  Unfortunately, since a vertical resolution of 480 pixels

would result in displaying only a small portion of a weather photo,

I also decided that the program would plot only one line for every

two received. Lines are not skipped, however: a smoothing algorithm

is used on every two received lines to produce a smooth-appearing

single line, which is then plotted.  Using this method, about

two-thirds of a complete weather photo can be displayed on a

640/480 screen.

Finally, the completely impartial(but interesting) news-- forget what

was said about white being the highest frequency and black being the

lowest.  In commercial fax receivers, those conditions undoubtedly

obtain.  However, I found that the best results were achieved when

receiving faxes on the other sideband; hence, WeatherMan employs a

reversed palette in which darker colors represent higher frequencies.

As will be seen later on, WeatherMan's configuration file can be

modified to comply with the "standard" if that's what you need.

[III] Connecting a Radio to a Sound Card

To use WeatherMan, you must connect the audio output of your radio

receiver to the microphone input of your sound card.  But first:

*****************************************

*                                       *

*     Warning!  Warning!   Warning!     *

*     Warning!  Warning!   Warning!     *

*     Warning!  Warning!   Warning!     *

*                                       *

*****************************************

Got your attention?  Good, because this is very important:  unless

you completely understand what you are doing when you connect any

two pieces of electrical equipment together :

THERE IS AN INHERENT RISK OF PERMANENT DAMAGE TO EITHER OR BOTH

DEVICES OR TO YOURSELF!!!!

For people fortunate enough to have a shortwave receiver equipped

with a mic-level output jack, connecting the radio to a sound card

is simply a matter of stringing a patch cord between the microphone

output of the receiver to the microphone input of the sound card.

I have an old, battery-operated Radio Shack DX-440 which I use for

wefax (yes-- it does have a mic output, albeit an undocumented one)

and this method works just fine.  But a warning-- if your radio is

powered from the AC line and you're not sure if the radio uses an

internal isolation transformer, DO NOT connect it directly to the

sound card, even if there is an output jack.  Use the method below

instead.

The easiest and safest way to connect a sound card to a radio is to

plug a microphone into the card's MIC input, then place the active end

of the microphone *very* close to the radio speaker.  Better still, put

the active end of the mike between the earcups of a set of headphones,

then plug the heaphones into the receiver's headphone jack.   Secure the

mike in place with rubber bands, use a little scrap foam rubber for sound

baffling and you're in business.

[IV] Using WeatherMan : Creating a Configuration File

Now that your radio and sound card are connected together, there's

one more thing you have to do before running WeatherMan: edit the

program's "configuration file", which holds essential details about

your hardware setup and display preferences.  A sample configuration

file is given below (it's a copy of the one supplied with the program

set):

1) 59676 <---------------------- sweep delay number

2) 1050  <---------------------- lowest audio frequency (1050)

3) 1700  <---------------------- highest audio frequency (1700)

4) $220  <---------------------- I/O port used by sound card

5) 5     <---------------------- interrupt used by sound card

6) 1     <---------------------- DMA channel used by sound card

7) 0      63      63      63 <-- <color number> <R> <G> <B>

8) 1      59      59      59      each RGB value must be >= 0 and <= 63

9) 2      55      55      55

10) 3      50      50      50

11) 4      46      46      46

12) 5      42      42      42

13) 6      38      38      38

14) 7      34      34      34

15) 8      29      29      29

16) 9      25      25      25

17) 10     21      21      21

18) 11     17      17      17

19) 12     13      13      13

20) 13      8       8       8

21) 14      5       5       5

22) 15      0       0       0

Let's go through the list (the line numbers, e.g., "20)", are for

reference only and should not appear in the actual configuration file):

Line 1 : "sweep delay number"-  This number is used to change the

"tick rate" of the PC's internal time-of-day clock so WeatherMan can

use it as a picture-synchronizing reference.  When the proper value for

this number is entered into your configuration file, received pictures

are properly aligned vertically with minimum right/left skewing.  The

function of this number is analogous to the "horizontal hold" control

found on older television sets.

How do you know what sweep delay number to use?  The number given in

the sample WXMAN.CFG file is only approximate and will probably not

be correct for your setup: you will have to determine the value you

need to use by trial and error.  Fortunately, I can offer some

guidelines:

Suppose you are receiving a fax composed of vertical lines that is

supposed to look like this:

|       |       |

|       |       |

|       |       |

|       |       |

|       |       |

but instead it looks like this:

\       \       \

\       \       \

\       \       \

\       \       \

\       \       \

In this case, the sweep delay number should be INCREASED by pressing

the plus ("+") key.  Note that "+" is entered by pressing the "=" and

SHIFT key simultaneously.

Conversely, if the picture looks like this:

/       /       /

/       /       /

/       /       /

/       /       /

/       /       /

the sweep delay number should be DECREASED by pressing the minus ("-")

key.

You should only have to adjust the sweep delay number once for a

particular sound card/computer combination.  When you exit

WeatherMan, you will be informed of the last sweep delay used by

the program.   Insert this number into your WXMAN.CFG file.

Lines 2 and 3: "lowest audio frequency" and "highest audio frequency"--

These define the white-to-black frequency range and have two principal

uses: 1) by widening/narrowing the range, you can stretch/compress the

contrast of the received picture, and 2) by  shifting the center of

the range up or down, you can possibly reduce interference caused by

transmissions on adjacent frequencies. Experiment with various

combinations to achieve the best results.  The numbers given are

in cycles per second (Hz).

Line 4: Sound card I/O port- If necessary, change this number in

accordance with your particular sound card installation.  Consult

your sound card's reference manual for details.  Also, note that

the prefacing "$" denotes a hexadecimal number.

Line 5: Sound card interrupt number- Change if necessary, as in Line 4.

Line 6: DMA channel used by the sound card-- Change if necessary, as in

line 4.

Lines 7 through 22: color palette- This is pretty self-explanatory.

The default palette is for a 16-level gray scale.

The format of the configuration file requires that comments be

separated from the file elements by at least one space. The file

should not contain any empty lines.

[V] Running WeatherMan

Once you have created a configuration file, WeatherMan is run using

the following syntax:

WXMAN

or

WXMAN <configuration file name>

If you choose the first option, "WXMAN", you must have a configuration

file with the name "WXMAN.CFG" located in the same directory as the

program file.  The second option lets you choose the name and path of

a different configuration file.  Suppose, for example, you have a

reversed-palette configuration file named "WXMANREV.CFG" in directory

"CONFIGS" located on drive C:.  You then run the program with the

command "WXMAN C:\CONFIGS\WXMANREV.CFG".

With your shortwave receiver turned on and connected to the sound card,

activate the receiver's BFO, then tune in a wefax station.  Adjust the

BFO up or down for the best picture.  When tuning the BFO, it's usually

easiest to start at a high-frequency audio output and work back to the

proper frequency.

[VI] WeatherMan Keyboard Commands

Once WeatherMan is up and running, you have several commands at your

disposal:

<C>lear Screen

<1>..<9> Delay Picture Reception

<BS>  (backspace) Nudge Picture Left/Stop Nudge

<P>osition Graphics Cursor

<G>rab a Picture

<H>elp with Commands

<ESC> Exit WeatherMan

<+/-> increase/decrease sweep delay

Each of the commands is activated by pressing the letter key within

the "<>" symbols. Case is unimportant.

<C>lear Screen

Pressing "C" clears the screen and the current image is

lost forever.  After clearing the screen, the picture will

probably have to be re-aligned (see below).

<1>..<9> Delay Picture Reception

Pressing "1" through "9" causes sampling of the incoming

fax to be delayed an amount of time equal to 1-9 "ticks"

of the re-programmed PC real-time clock.  Each tick represents

an increment of one-tenth of the horiziontal screen width.

These commands let you "walk" the true edge of the incoming fax

so that it's close to the left side of the screen.

<BS>  (backspace) Nudge Picture Left/Stop Nudge

The "1" through "9" keys move the picture to the left only in

discrete amounts, so it's possible that you could move a picture

only to find it doesn't completely align with the left edge of

the monitor screen.  In such cases, pressing BS will cause the

picture to start skewing to the left automatically.  This key

is a toggle switch-- press it once to start the automatic shift

and when the picture is aligned to your satisfaction, press it

again to stop the motion.

<P>osition Graphics Cursor

After positioning a picture with the "1" through "9" keys,

and/or the BS key, pressing "P" moves the graphics cursor to

the top line of the screen, without clearing the screen.

Previously displayed data will be over-written.

<G>rab a Picture

Pressing "G" saves the currently-displayed picture to disk.

The saved picture is time- and date-stamped at the bottom of

the screen.  Saved picture files are given the name

"GRABxxx.RAW", where "xxx" denotes the number of the saved

picture.  When WeatherMan starts up, it searches the current

directory for previously-saved pictures and sets its first file

name to the next highest number to avoid overwriting old files.

When a picture is saved to disk, the timing relationships in

the PC become somewhat unsettled for fax reception.  When this

occurs, you'll notice the new data being plotted will start

skewing fairly rapidly to the right.  After a few such lines

are drawn, the picture will again become synchronized.  You

can hasten re-synchronization by pressing BS twice, or by

clearing the screen.

<H>elp

Pressing "H" displays a command summary.  No fax data is

received or displayed while the "H" command is in effect.

Press any key to exit the help screen.

<ESC> Exit WeatherMan

Pressing the escape key unconditionally exits the program;

you are not prompted to save the picture first.

<+/-> Increase/Decrease sweep delay

These keys are used to adjust the sweep delay, as explained

above.

[VII] The SHOWPIC program

Having saved a bunch of pictures to disk, it would be nice to see what

they looked like again, wouldn't it?  That's the purpose of SHOWPIC

and its syntax is

SHOWPIC [-p] <picture file> <picture file>...

where the optional -p switch loads a palette from the file SHOWPIC.PAL.

The format of SHOWPIC.PAL is the same as in lines 7 through 22 of the

configuration file.  See the sample palette file for details.

Example 1:

SHOWPIC GRAB0.RAW GRAB11.RAW GRAB2.RAW

displays the pictures GRAB0.RAW, GRAB11.RAW, and GRAB2.RAW consecutively

using the built-in gray-scale palette.

Example 2:

SHOWPIC -p GRAB1.RAW TEST.RAW

displays the two given pictures using the palette contained in SHOWPIC.PAL.

You can move to the next picture in the set at any time by pressing any key.

WeatherMan's RAW picture file format is exceedingly simple, consisting

of a file of 640*480=307,200 bytes, with each byte representing a

color number in the 0 to 15 range. The file is a record of consecutive

pixel values starting at the upper left-hand side of the screen (0,0)

and extending to the lower right-hand side (639,479).  This format

should make it easy for you to write your own programs for manipulating

captured picture data.

The utility program RAW2PCX.EXE (see section [VIII]) can be used to

convert the RAW format into the PCX format used by many PC

paint/image-processing programs.

[VIII] The RAW2PCX program

To convert a picture file to a PCX graphics file use either of the commands

RAW2PCX  <input file name>, or

RAW2PCX  <input file name>  <output file name>

Example 1:

RAW2PCX GRAB0.RAW

This command creates a new file "GRAB0.PCX" in the PCX format.

Example 2:

RAW2PCX GRAB0.RAW  TEST.PCX

Same result as in Example 1, but the output file is named "TEST.PCX"

instead.

Warning: RAW2PCX will overwrite any existing file having the same

name as the output file.

*****

[IX] WXDEMO.EXE

Wefax broadcasts have a very distinctive sound and once you've heard

one, you'll probably never forget what it sounds like; unfortunately,

the sound is difficult to describe in words.  That's where the WXDEMO

program comes in.

Syntax:

WXDEMO <picture file>

WXDEMO takes a single argument, the name of a picture file, then

simulates the reception of the picture through your sound card and on

your monitor.  If you've never heard a fax transmission before, WXDEMO

should help you learn what to listen for as you tune through the

shortwave bands.  Keep in mind that this simulated reception will be

noisier than the original signal-- this results from approximating

the original 8-bit audio values by the 4-bit pixel values.

[Appendix 1] Weather Facsimile Frequencies.

The following is a very small list of stations that transmit weather

photos and charts during the day.  It is not at all inclusive: there

are literally dozens of other stations across the HF bands.

(All frequencies are in kilohertz.)  If you have Internet access, drop

me a line at paulhtc@netcom.com and I'll be happy to send you a larger

list.

Frequency (KHz)     Location

---------          ---------

6453             California

8682             California

9090             California

11090             Hawaii

16135             Hawaii

8080             Virginia

15959             Virginia

20015             Virginia

[Appendix 2] Troubleshooting Q&A

What WeatherMan is and isn't:

WeatherMan is a program to decode and display radiofax signals, nothing

more and nothing less.  When presented with a high-quality, low-noise

signal, the program will reproduce a fairly good image of the transmitted

picture.

WeatherMan does not, in any way, enhance the receiving capabilites of

a shortwave receiver.  In fact, the opposite is probably true-- Computers

and their peripherals are notorious for generating broad-spectrum

radio-frequency interference.  It is entirely likely that you will

not be able to receive some stations while the computer is running,

even stations that would otherwise be received just fine.

Several things can be done to improve reception:  Locate the radio and

its antenna as far from the computer as possible.  Use a high-output

microphone.  Use a good long-wire antenna.   Don't use a light-dimmer,

electric motors, or fluorescent lights near the radio or its antenna.

Pray that the ionosphere cooperates-- all the usual stuff.

I should also mention the possibility of taping fax transmissions

while the computer is off for decoding and displaying at a later time.

While common audio tape decks have insufficient speed regulation

to be very useful in this regard, some modern VCRs are also designed

to function as high-quality audio recorders.  (I have a Toshiba VCR that

I've used for just this purpose.)  If you decide to try this, you'll

probably have to change the sweep delay number to achieve good results.

VCRs are fairly precise instruments, but they're not always accurate

with regard to playback timing.

Now on to the questions.

Q: Why isn't anything being displayed?

Make certain your radio is turned on and the BFO is activated.

Check the connection between the radio output and sound card mic

input. If you have an SBPro, an SB16, or a MediaVision PAS16 card,

use the appropriate mixer command to set the microphone level to

maximum sensitivity before running WeatherMan.

Q: Why does the picture look like a photo negative?

Either you have loaded a reversed palette in the configuration file

or you are tuned to the wrong sideband.  Adjust the file or the BFO

accordingly.

Q: Why is the picture grainy?

The signal may be weak, or there may be a source of interference nearby,

such as an electric motor, fluorescent light, or the like.  Use a good

antenna and/or increase the mic input level, and/or remove the source

of the noise.

Q: Why is the picture "ghosty"?

Just like television, if a signal is received along two or more paths

having different effective lengths, you'll get ghosts.  There's no

cure, except perhaps to re-orient the receiving antenna.

Q: Why are there 30 wavy vertical lines across the picture?

You're picking up 60 Hz (power line) interference from somewhere,

possibly from a ground-fault loop.  Try transformer-coupling the

receiver to the sound card and make sure you're using a good quality

shielded cable.

Q: I've set the "sweep delay" constant.  Why do my pictures still come

out slightly skewed at times?

Because of variations in sound card sampling rates and since the PC's

internal clock has a finite resolution, it's unlikely that your pictures

will come out perfectly square.  However, any skewing in which the

picture is off by more than a few pixels is an indication that the

sweep delay constant is not properly adjusted.

Q: I get great reception until I turn on the computer.  What gives?

The computer is a wonderful source of radio waves.  Keep the antenna

as far as possible from the computer and connect it to your radio

with coax.  The coax shield should be connected to a good ground.

Q: When my radio is on, why do I hear Skippy the Clown urging me to

do bad things?

Uh huh... That's kind of out of my area of expertise...

[Appendix 3] Supported Sound Cards

WeatherMan has been tested extensively using the following sound cards:

1. Sound Blaster 1.5

2. Sound Blaster Pro

3. Sound Blaster 16

4. ProAudio Spectrum 16 (PAS16)

The program *may* run on other "100% Sound Blaster compatible" cards,

but I can't guarantee that it will.  At the present time, I intend to

support only the cards listed above.

[Appendix 4] About the Sample Pictures

The sample pictures were received using the following extremely

non-ideal setup:

Radio      : Radio Shack DX-440

Sound Card : ProAudio Spectrum 16, Sound Blaster Emulation

Antenna    : 25-ft. random wire antenna, located indoors

Location   : Third floor of an apartment building

Problems   : 1) antenna only 6 ft. from computer

2) power company transformer about 50 ft away

from computer.

Even with the setup above, I think the pictures are pretty good; I'll

leave it up to you to imagine what they might have been like using an

excellent shortwave receiver and a good antenna.

***************

Thanks for trying WeatherMan!