CWReader

A program that reads CW from el-bug to your log-program

version 1.0

Made by Mikko Noromaa OH3LIM

Reading this document is necessary for a thorough understanding of

the operation of this program. And a thorough understanding is

necessary to be able to use this program successfully and

efficiently.

However, prior to reading this rather long document, you might want

to view the file READ.ME. If you want, you can print the READ.ME by

typing "COPY READ.ME PRN" and this document by typing "COPY

CWREADER.DOC PRN".

WHAT IS CWREADER?

CWReader is a program that copies morse-code from your electronic

keyer (el-bug) to your log-program. It's mainly intended for use in

contests, and may be used on a DX-pedition as well. CWReader can be

configured to work with almost any kind of a program, so you'll be

able to enter DOS-commands with your keyer, for example! If you

send CW very fast and type with the keyboard very slowly, you may

also find it convenient to use your keyer to 'write' text to a

word-processing program. At last, CWReader's capabilities are

largely dependent on your imagination!

Even if you wouldn't want to connect a keyer to your computer, you

can use CWReader as an enhancer of some contest logging-programs.

It provides a CW-speed range of 12 to 50 WPM with all logging-

programs that otherwise don't support such a large range. CWReader

also allows you to define 'trapped-keys', with which you have the

power to entirely change the actions of some keys on the keyboard.

THE ACCOMPANYING CONFIGURATION PROGRAM, CWRCONF

For reasons of space and efficiency, no configuration options are

included in the CWReader-program itself. Instead, all configuration

is done with a separate program, called CWRCONF. This program

allows you to configure almost everything you could ever imagine

you wanted to change (and more)! The options range from setting the

ports and pins where the keyer is connected, through setting all

the keys used to operate CWReader to redefining all the CW-

characters.

The options are divided into two classes: global options and

contest program-specific options. Global options include settings

that are the same, no matter what contest-program you use with

CWReader. The keyer-connection, initialization-key and default CW-

speed are examples of global-options.

The contest program-specific options include options like the keys

used to change the CW-speed, the trapped-key definitions and the

sequences to issue in the initialization. The contest program-

specific options also include the global-options, which means, that

although the global-options are intended to be 'global', you can

override some (or all) of them as necessary for a given logging-

program.

All the options are saved in files that have an extension of

".CWR". The global options reside in USERCFG.CWR and the contest

program-specific options in their own .CWR-files, the name usually

being the same as that of the contest-program for which they are

intended. CWRCONF manipulates these files and CWReader loads them

into memory upon startup.

A complete list of all the options available for configuration with

CWRCONF is given later in this document, along with a thorough

description of the options.

INSTALLING CWREADER

CWReader is installed (loaded in memory) by running the program

"CWR.COM". If you give no parameters to it, it assumes that what

you want to load is the configuration for CT-program, found in a

file "CT.CWR". If you give a command-line parameter to CWReader

(for example, you type "CWR OWNPRG"), CWReader loads the

configuration file "OWNPRG.CWR". If this file (or in the default-

case, file CT.CWR) doesn't exist, you'll get an error-message. As

the file USERCFG.CWR is also loaded at startup, it must also exist

in the current directory.

If you don't have these .CWR-files, you can create the default ones

by running CWRCONF. When it detects the absence of these files, it

creates them and places the default-values in them. If you don't

want to change these defaults, you can quit CWRCONF immediately,

and you'll be ready to run CWR.COM and get it installed in the

memory.

That was the first step needed to get CWReader 'up and running'.

Now CWReader is loaded in memory, but it's not active yet. Your

next step is to load the contest-program you are going to use and

for which the configuration-file you loaded is intended. Once you

have the program ready and running, you can initialize CWReader by

pressing Alt-I (or the keys you set with CWRCONF). CWReader issues

the initialization sequences set with CWRCONF and after it is done,

beeps briefly to mark succesful initialization.

Now CWReader is ready to use with your contest-program. When you

press the initialization key (Alt-I by default) again, CWReader

uninstalls itself, this time beeping a little longer to mark

succesful uninstallation. CWReader is not, however, completely

wiped out of computer's memory, but is, in a way, simply suspended.

You can reinitialize it again by pressing the initialization key

once more.

When you exit the contest-program you were using, be sure to

uninstall CWReader first! If you can exit the program using some

key-combination (like Alt-X or Alt-Q), you'd better do so, rather

than exit by choosing "quit" from a menu. This allows you to set an

'uninstall-key' with CWRCONF. When you press this uninstall-key,

CWReader automatically uninstalls itself and then passes the key to

the contest-program. (note these two differences when compared to

Alt-I operation!)

Should you accidentally exit your contest-program without

uninitializing CWReader, the system will crash. Most probably even

the contest-program won't quit like normally and when you try to

run other programs, the computer is very likely to hang. To prevent

loss of data and other damage, you should always boot your computer

after this kind of an accident!

By running CWR.COM again when CWReader is already installed in

memory, you can update the options with which CWReader is working.

For example, when you first simply type "CWR" and load the

configuration for CT that way, you can later type "CWR OWNPRG" to

change the configuration options for those of OWNPRG. Or, you may

do changes to the global-options with CWRCONF and then run CWR.COM

again to get these options in effect.

There is no way to completely remove CWReader from the memory, but

if you want to do so, use utilities such as MARK and RELEASE. There

should be numerous of such utilities in the public domain and

shareware-market. What you should avoid, however, is to remove

CWReader from the memory (by RELEASE, for example) without first

having uninstalled it. If you do so, your computer's clock will

begin running about four times as fast as normally! When you boot

your computer again, the time will be reset according to the

computer's battery backed-up real-time clock.

CONNECTING YOUR KEYER TO THE COMPUTER

With CWRCONF, you can configure the keyer to any port (LPT1-4,

COM1-4) and any pin of the selected port. Here, however, we make

the assumption, that you are connecting your keyer to a LPT-port

and to it's SLCT-pin (pin number 17 in DB25).

Basically, the task is quite simple: just connect the keyer so,

that whenever it keys your rig, it will connect the SLCT-pin to

ground. Think about connecting a straight key to the computer: it

would be simply connected between the SLCT-pin and the ground. The

idea is essentially the same when connecting a keyer of any kind.

Using the accompanying program CWRTEST

The CWRTEST-program is intended to help you to connect your keyer

to the computer. Follow the on-screen helps to select the port

where your keyer is connected (use 'C' to COM, 'L' for LPT and

numbers 1-4 to select the individual port). You'll see the current

bit pattern of the selected port and whenever a bit's state

changes, it's shown on the screen.

Use the arrow-keys to select a bit and press ENTER to attach a

sidetone to it. Press ENTER another time to toggle the 'polarity'

of the sidetone (i.e. is 1 ON and 0 OFF or 0 ON and 1 OFF). If you

hear a continuous beep when starting CWRTEST, press ENTER to change

the polarity and stop the beep. Now, if you have connected your

keyer correctly, you should hear your sending from the computer's

internal speaker. Confirm that it sounds OK.

When using CWReader and a log-program to send CW, the wirings

between your computer, rig and keyer should be as follows:

������������ͻ      �����ͻ         �����ͻ

�  COMPUTER  �      � RIG �         �KEYER�

�  in    out �      �  in �         � out �

������������ͼ      �����ͼ         �����ͼ

�      �            �               �

�      ���������������������#������Ĵ

�                                   �

�������������������������������������

The diode (��#�) between the keyer and rig is necessary, because

without it, there would be a direct way from computer out to

computer in and CWReader would copy what the log-program sent!

The existence of the above problem can be checked with CWRTEST by

pressing the SPACE. This causes CWRTEST to send a little message

through the selected port and if you hear it from the speaker, you

should take actions to correct the problem.

LPT-interface versus COM-interface

If you have the choice, use the LPT-interface. It is significantly

the simplest and the most reliable way to connect your keyer to the

computer. However, if you have no choice but to use a COM-port, you

should take the following facts into consideration.

First of all, the pins of a COM-port are normally in the 0-state,

which means that they are at the same potential than the ground.

Clearly, there is no point in connecting them to the ground through

you keyer. The way I have succeeded in solving the problem was to

connect the interface-pin in question to a pin, which was

constantly held in the 1-state. The pins DTR and RTS are usable for

this, but if CW is output through the same COM-port, these pins are

reset to 0 by the CW-keying routine. And holding them in the 1-

state causes the keying line to the rig being held active all the

time!

Anyway, if you configure (with CWRCONF) the keyer (or external-

disable) to a COM-port, this port's DTR and RTS are set active by

the initialization routine. However, they may be reset back to 0 by

a CW-keying routine or just for some other other reason. Careful

experimentation is needed, if you are going to connect your keyer

to a COM-port!

THE DEFAULT SETUP OF F2, F6, F7 AND INSERT

The following paragraphs describe the default setup of the F2-, F6-

, F7- and INSERT-key when using CWReader with CT. You may alter

this with CWRCONF and the description as such serves as an example

of the power of configuring CWReader. You don't need a keyer

connected to the computer to use the information contained here.

A problem with the CT program is, that when you are answering CQ's,

the message you get from F2 is simply of form "599 123", i.e. there

is no possibility to get a leading "TU" or "R". CWReader offers a

cure for this. It's setup so, that when you press the F2-key, what

is actually passed to CT is F7 followed by F2. When the message

programmed in F7 is "TU 599" and the message in F2 is simply "#"

(the serial-number) or your zone, the output is effectively of form

"TU 599 123". When you normally press the INSERT-key, CT sends

first the other station's call (F5) followed by the exchange (F2).

CWReader changes the action of INSERT so that it passes the keys

F5, F6 and F2 to CT. And when the message in F6 is "599", the

result is of form "<call> 599 123".

The abovementioned feature is a significant improvement to CT's

operation, at least in author's opinion.  The drawback of it is,

however, that you effectively lose the CW-memories F6 and F7. So if

you are used to using them (sending "?" and "CL?", for example) you

could consider removing these trap-key settings from CT.CWR with

CWRCONF.

THE 'INTELLIGENCE' OF CWREADER

This section explains how CWReader interprets some characters,

words, spacings and combinations of them. You should know this

information in order to send CW in the most optimum way for

CWReader. In the discussion below, the contest-program is assumed

to be CT. (note that all of these features can be enabled and

disabled with CWRCONF)

Receiving a 599-report:

When you send a station's call with your paddle, it is copied

to the CT's call-field. Then, after the call, you immediately

send the report, "5NN", at which point CWReader wakes up. It

deletes the characters "5NN" after the call from the call-

field, moves the cursor to the number-field and commands CT

to send the exchange-number.

Sometimes you may send 599, by mistake, as "HNN". But don't

worry, CWReader detects this as well and actually handles

"HNN" just like it would "5NN".

CWReader's 599-detection is not always active. You can, for

example, work station VK5NN not having to worry about

CWReader thinking the "5NN" part of the call was a report.

The logic CWReader uses is simple: begin monitoring for 599

after at least one number and at least one letter have been

received. However, if the number is the first character of

the call, begin counting from the second character. There is

one exception: when a question mark is received, 599-

detection is immediately enabled. This makes sending the

following call impossible: "V?5NN", so you should probably

use question marks only at the end of the partial call.

After receiving 599, there is a little pause in copying

characters from the keyer. This pause is there so that you

can manually send the exchange number, if you, for example,

prefer to use your keyer's serial-number logic instead of

CT's. Be careful, however, when you make a mistake in sending

the number and keep a too long pause before sending an error

character: the whole log-line, including the call, will be

deleted!

Receiving TU:

Imagine you have just finished entering the number the other

station gave you and are ready to confirm the QSO. Provided

you do not use the keyboard to do this, you will send

something like "TU OH3LIM" with your paddle (or perhaps

simply "TU", if the pileup is heavy). CWReader can detect

both of these cases and take actions accordingly.

Let's begin with the reception of a simple "TU". After

receiving this character combination and a resonably long

space, CWRreader deletes the TU from the log and confirms the

QSO like you would do it with ENTER. The delay after TU is

necessary, because otherwise calls like N2NTU would be

seriously mistaken. For the same reason there must also be a

resonably long space BEFORE receiving the TU or otherwise it

will not be acted upon.

If you send your own call (set with CWRCONF) after the "TU",

CWReader doesn't even print the call on the screen: if it

appears to be your call, the action is same as TU's alone

(without the delay at the end, however), but if it is not,

the call is displayed and no actions are taken. For example,

with a call like "N2TUO" (and if your call began with an

"O"), the last "O" would not display immediately when you

send it, but later after the 5 of 599 has been sent. When the

TU is followed by your call, there need not be the space

neither in the beginning nor in the end of the sequence.

TU is sometimes given without the space between T and U,

making it effectively X. From CWReader's point of view, the

TU can be given as X and all actions apply to it as well.

However, it's not suggested that you make it a habit giving

TU as X, but just in the case it happens to slip from your

paddle...

Skipping CQ-calls

Although you should use CT's F1-key to send CQ-calls, it may

sometimes be necessary to send a CQ with the paddle. When

CWReader detects the sequence "TEST", it erases it from the

screen and does not write anything you send afterwards on the

screen. When CWReader detects another "TEST"-sequence, it

resumes normal operation and write everything following the

sequence on the screen. Be sure to end your CQ's in "TEST"

(or as set with CWRCONF), because otherwise you'll

effectively disable CWReader!

If you make a mistake in sending the CQ and send an error-

character (as described in a later section), CWReader's TEST-

mode is reset. That is, all you send after the error-

character, will be put on the screen. Note, that the error-

character causes its normal action to be executed (normally

the deletion of the current field)!

Entering calls partially by keyboard and partially by paddle

Say you are not a very fast typewriter. When a station is

calling you, you can only enter a few characters of his call.

Say there is EI5ISF calling you and you enter EI5 after which

you must hurry to your paddle to send, NOT the rest of the

call, but the full callsign, of course. CWReader starts

putting the characters on the screen when you have got past

the characters that are already on the screen. If you make a

mistake, however, like sending ES5 instead of EI5, the

mistaken character will be put on the screen where the cursor

points! That ES5 would make it look like EI5S on the screen,

then. In this case, the best action is probably to send an

error-character (and get the whole call wiped out), as

discussed in the next section.

Whenever you press a key on the keyboard, CWReader resets its

'current callsign position'-variable, making it impossible

for you to first enter EI5 by keyboard, then send EI from the

paddle, then enter I (or IS or ISF) by keyboard and lastly

send 5ISF from the paddle. That is, you should not enter more

of the callsign AFTER you have begun sending with your

paddle! Just let CWReader read the callsign directly from the

paddle for you.

Receiving error-characters

CWReader knows three kinds of error-codes: the standard

error-character (...-.), a bunch of dots (........) and a few

dots sent far-spaced. The error-character and bunch of dots

are easy ones; just make sure you send at least 6 dots, since

5 would result in receiving the number '5'.

A minimum of three far-spaced dots is needed to interpret it

as an error-code. There is no maximum, but when you keep a

'long' pause (compared to the spaces between other dots), the

error-code is considered ended. Note that it's impossible to

enter a call like JJ0EEE from the paddle! However, a call

like EA1XX is possible to enter, but you must remember to

keep a long enough pause between the last error-dit and the

callsign's "E". Trying it out is the best way to make your

timing correct!

Remember, that when an error-character is sent, the WHOLE

log-line is emptied. This corresponds the keyboard-entered

Alt-F8 (can be changed with CWRCONF, though). Make sure you

have the stations callsign firmly in your head when having to

issue an error-character!

ISSUES ABOUT CW-SPEEDS

There are two CW-speeds in effect: the one of your contest-program

and the one of your keyer. In the discussion below, we'll assume

the contest-program to be CT. It applies, however, to every other

contest-program as well, except that the keys used may be different

(all of them can be changed with CWRCONF).

CT's speed is changed by using the keys Alt-F9 and Alt-F10 (Alt-V

is disabled) and keyer's speed changing is dependent on the keyer.

CT's speeds range from 12 to 50 wpm (yes, CWReader extends it to

this range), and your keyer may further extend it considerably

(both below 12 wpm and above 50 wpm).

You should be very careful when changing your keyer's speed. First

of all, you should not change it too much DURING the sending of a

character. Second, you should not change the speed to less than

half the original speed at once. If you want to do such a large

speed change, do it in two phases sending something between these

two (not an E or T, however). Alternatively you can hit two times

Alt-D after you've changed the speed to reset CWReader's speed-

expectations. Third, you may not change CT's speed during the

sending of a character. Especially if the change is large, CWReader

may copy one or more characters wrongly.

You can change the speed upwards as much as you like at once. But

there is a limit of maximum copyable speed, which is a function of

the speed set in CT! If CT's speed is at minimum (12 wpm), you

should not try copying speeds greater than 24 wpm. However, because

12 wpm is such a low speed, this shouldn't be a handicap, because

with CT's speed set to 20 wpm, you'll be able to copy speeds of 40

wpm. And if you set CT's speed to the maximum, 50 wpm, CWReader

will be able to copy speeds up to 100 wpm! The station trying to

copy you may have a little trouble, though.

The above statements clearly suggest that you should keep CT's

speed and you keyer's speed 'in sync'. If you lower CT's speed,

you'll most probably want to lower your keyer's speed as well.

Therefore, there is no problem in practice, but it's good for you

to know that conflicts in speeds may lead to errorneous operation

of CWReader.

After having changed the keyer speed considerably, avoid sending

single-element characters 'E' and 'T'. As first characters, these

may be copied incorrectly. If you begin getting T's only,

regardless of what you send, the keyer's speed has been changed

down too rapidly. You can either raise the speed and then bring it

slowly down again or press two times Alt-D.

If you break your keyer sending its memory by tapping the paddle

(or by any other means), you run into a terrible risk of receiving

one very short element (the interrupted element), which causes

CWReader think your keyer's speed has raised dramatically. Usually

this causes everything that follows to be copied as T's. If you

must break like this very often, you'll sure need to learn to use

Alt-D!

One very important thing about CWReader is, that your contest-

program's CW-speed should ALWAYS be kept at 22 WPM! By default,

configuration for CT sets this upon initialization and after that

it's impossible to change the speed. Alt-F9 and Alt-F10 are trapped

by CWReader and Alt-V is discarded by CWReader, so there remains no

way to change CT's speed. That's how you should configure any other

contest-programs you may use with CWReader.

The explanation for the above follows (don't worry if you don't

understand it). When first loaded, CWReader chains itself to the

timer-interrupt and when the contest-program (called CT from here

on) is loaded, it adds itself to this chain. Sending CW is

accomplished by means of the timer-interrupt, as is receiving CW by

CWReader. Because CWReader needs much more precision for the

receiving as CT needs for the sending, it sets the timer-interrupt

to occur four times as often as CT requires. The portion of

CWReader (in timer-interrupt chain before CT) then passes only one

of four interrupts to CT so it seems to CT as the timer were

running at the normal rate. When the CW-speed is

increased/decreased by CWReader, the timer-interrupt's rate is

still held high, but if CT gets to change its CW-speed, it drops

the rate out of the four-times-as-large -value. This means that

copying does not work anymore and that CT's CW-speed drops to

fourth the intended!

EXAMPLE WORKING CONFIGURATIONS

This sections is to show you different ways in which you can

benefit from CWReader. Of course there are numerous other ways as

well, so feel free to adapt your own working-practice either as

such or partially modified, to work with CWReader. Take the

'configurations' in this section only as examples!

'The best way' to use CWReader

'The best' here means most trouble-free and reliable

operation with CWReader. Of course this depends largely on

your paddle-sending skills, but with this method, the errors

should be minimal.

The key issue is to use your keyer's memories as much as

possible. When you pick a station's call, send it from your

paddle, then push keyer's button 2 to send the 599 (you can

make mistakes in it too, especially after 40 hours of

continuous operation). If the contests calls for fixed-

numbers (like zones, power or age), you can include this

number in keyer's message and program CWReader not to send

anything after 599 (with CWRCONF). You should not use your

keyer's serial-number logic, because this may create

conflicts between the numbers you send and the numbers CT

thinks should be sent.

After the QSO is complete, you press keyer's button 3 to send

"TU <owncall>" or button 4 to send only "TU" (in case of a

heavy pileup). CWReader confirms the QSO for you and you are

ready to log another.

Doing it without a memory-keyer

Without a keyer capable of storing to and sending from

memories, you are not very well off, actually. Making errors

in sending 599 or TU can be quite 'disastrous', meaning that

you must do some work to correct your errors by the keyboard.

But if you are skilled in sending CW, there should be no

problems, as your sending will be much like it came from

memories, right?

In the case of a non-memory capable keyer, you should confirm

QSO's by using CT's plus-key. This way you will minimize the

error possibilities. If you want to send only "TU" without

your call, you can do it with your keyer. Remember, that "X"

is interpreted as "TU" and "HNN" as "599" and you may

configure "...... -. -." to be received as "599" as well!

Using CWReader without a keyer

Yes, this is possible also. There is, however, not very much

you can do with CWReader without a keyer. The only thing is

the enhanced action of F2 (sending "TU 599 123" instead of

"599 123"), so if you consider this a worthwhile enhancement,

go ahead and install CWReader. Remember, however, that F6-

and F7-memories can't be used for your purposes when CWReader

is installed.

OPTIONS CONFIGURABLE WITH CWRCONF

The global options (saved in USERCFG.CWR)

Monitor type (default COLOR)

This can be toggled between COLOR and MONO. If MONO is

selected, the screen-buffer is assumed to be elsewhere and

the cursor-position is read differently. Set this to COLOR

even when you are using a color-monitor in a mono-mode!

Your callsign (default OH3LIM)

Enter your callsign here (the maximum length is 15

characters). This is used to check whether your callsign is

received after a "TU". If you do not use this feature, it

doesn't matter what you enter here.

Keyer connection (default LPT1, ACK-pin (0=ON)

This options allows you to set the port, pin and polarity to

specify where your keyer is connected. If there is no keyer

connected, select NONE for the port.

Ext. disable conn. (default NOT CONNECTED)

See above.

The initialization key (default Alt-I)

This specifies the initialization key, which is pressed to

initialize CWReader after loading a log-program. The

initialization key is also used to uninitialize CWReader.

The disable key (default Alt-D)

This specifies the key used to disable CWReader. When it is

pressed, nothing is copied from the keyer until it is pressed

another time. Then copying is continued and all internal

variables (speed expectations, for example) are reset.

The exchange to send after receiving 599 (default #)

This is the exchange that is programmed in the F2-key. This

exchange is played after 599 has been received from the keyer

and when you press the insert-key. With CT, enter "#" here if

you want a serial number to be sent. The maximum length of

the exchange is 8 characters.

The time to wait after receiving 599 (default 16 dits)

This specifies the delay (in dits of current CW-speed) after

receiving 599 during which nothing is copied from the keyer.

During this time you can manually send an exchange with the

keyer, for example.

"599"-detection (default ON)

Specifies whether the 599-detection logic is ON or OFF.

"TU"-detection (default ON)

Specifies whether the TU-detection logic is ON or OFF.

"TU <call>"-detection (default ON)

Specifies whether the "TU followed by your call"-detection

logic is ON or OFF.

"CQ TEST"-detection (default ON, TEST ... TEST)

Specifies whether the CQ-detection logic is ON or OFF and if

ON, what kind of a CQ-message do you use. The CQ-message is

specified in terms of start ("TEST", "CQ TEST" or "CQ <space>

TEST, if copying spaces is enable) and end ("TEST", "K" or

"<space> K"). Note, that if your callsign contains a K, you

will only be able to end your CQ with "TEST"! Also, it's not

possible for CWReader to understand a CQ like "CQ TEST DE

OH3LIM OH3LIM".

Interpret "X" as "TU" (default YES)

Specifies whether TU can be sent as an X. This can be set to

NO if you send the "TU" from your keyer's memory (or use log-

program to send TU's).

Interpret "HNN" as "599" (default YES)

Specifies whether 5NN can be accidentally sent as HNN and

still be understood by CWReader. This can be set to NO if you

send the 599 from your keyer's memory.

Action on 'bunch-of-dits' type of error (default Clear call)

This defines the action to take when receiving a 'bunch-of-

dits' type of error-character. If set to None, the error is

simply ignored. If set to Clear call, the log-line is wiped

out (by an elsewhere configured keystroke, by default Alt-

F8). If set to Reset variables, some internal variables are

reset, namely ones affecting 1) how far we are in receiving

599, 2) how many characters are already copied, 3) speed

expectations and 4) is a CQ-message in progress.

Action on '...-.' type of error (default Clear call)

See above.

Action on 'three-slow-dits' type of error (default Clear call)

See above.

Initialization delay (default 980)

The initialization process of CWReader sets the contents of

some CW-memories (F2, F6 and F7, specifically). On a floppy-

disk equipped computer this may take quite a while and

because the next initialization step may not begin before the

first one is completed, you may need to increase this delay

in order to get CWReader initialize itself properly.

With CT, at the initialization, you should see the "Set Fx-

message"-windows on the screen. If (especially the first)

doesn't appear, it's likely that this delay is set too low.

After initialization, you can check the messages of F2, F6

and F7. They should contain "<exchange specified by you>",

"TU 5NN" and "5NN", respectively. If they do not, try

increasing this value.

The values are specified in milliseconds, but they are

automatically rounded to the nearest 18.2's part of a second.

Default CW-speed (28 wpm)

This sets the default CW-speed which is put in effect every

time CWReader is initialized.

<THE CW-CHARACTER SETTINGS>

This selection brings up a new screen, in which you can set

how every possible CW-character is copied. All common

characters shorter than seven elements are displayed as such

and other characters can be set up as user-defined CW-

characters, which can be at most 15 elements long.

Press ENTER on normal CW-characters to change them and use

INSERT/DELETE to add/delete user defined CW-characters. When

asked to hit the key to assign, you can use Ctrl-C to cancel

the assignment, Ctrl-X to specify NONE and Ctrl-Z to make a

key-sequence. If you want the character ".-", for example, to

expand just to an 'A', you would simply press the 'A'. But if

you wanted it to expand to "ABCDE", you would hit Ctrl-Z, the

sequence ('A', 'B', 'C', 'D', 'E') and then end it by hitting

Ctrl-X.

You exit the CW-character settings -screen by pressing F10 to

accept or ESC to cancel all changes you have made.

Contest-program specific options

After selecting this choice from the main-menu, you are given a

list of all contest-programs CWRCONF knows (for which it found a

.CWR-file). You can continue by selecting one of them or by

selecting <NEW CONTEST-PROGRAM> which lets you add options for a

new contest-program. The options are initially set as for CT.

After this you are presented with the following options to change:

Program name

Specify the contest-program's name here. It's used in the

previous menu to let you know which options are configured

for which contest-program.

CW-speed display (default 70,9 ; 113 ; WPM)

Selecting this allows you to specify how CWReader will

display the CW-speed on the screen. Usually you want the

speed to be displayed in the same position as it is display

by the contest-program.

First you specify the X-coordinate (0-79) of the speed-field,

then the Y-coordinate (0-24). Enter 255 as X-coordinate to

specify NO DISPLAY. After the coordinates you are asked the

foreground and the background color. You may have to

experiment with these to find the right ones for your

particular contest-program. The last item you choose is

whether the speed is display in WPM (words per minute) or in

CPM (characters per minute).

X-coordinate of the call-field (default 15)

This specifies the X-coordinate of the callsign-field in your

contest-program. It's used when you send a callsign of which

you have already typed a part in the call-field. If you don't

need this feature, enter 255 here.

Keys used to change CW-speed (default Alt-F9, Alt-F10)

This specifies the keys you want to use to change the CW-

speed. Usually they should be set to be the same as in the

contest-program you use, but this is not constraint. First

you are asked for the speed-down -key, then the speed-up -

key.

Key, which shall be discarded (default Alt-V)

As explained earlier, your contest-program's speed must

always be set to 22 wpm. If it's changed, CWReader will

probably begin working errorneously. Therefore, it's

important, that the speed is not changed accidentally, and

with this option you can set one key to be totally discarded

to prevent that from happening. In CT, Alt-V is a way to

change speed in a way which CWReader can't duplicate; that's

why Alt-V is a good candidate for being discarded.

Clear-key (default Alt-F8)

This sets the key, which is used to clear the call-field.

It's used when an error-character is sent, and it's action is

set to Clear call. You might also use the backspace-key here.

Key used to advance to next field after 599 (default Space)

After receiving 599, CWReader should move the cursor to the

next field (namely the exchange-field) and this option

specifies the key with which it's done.

Key to send after receiving 599 (default F2)

After receiving 599, CWReader can command the contest-program

to send your exchange (HIGHLY recommended when the exchange

is a serial number). This option specifies the key with which

this is done. Enter Ctrl-X to send nothing after 599.

Confirmation key; when to reset variables (default '+')

This specifies the key with which you confirm a QSO in your

contest-program. When it's hit, most of CWReader's internal

variables are reset, so that it's ready for a new QSO.

Key, upon which to uninstall (default Alt-X)

Normally, you uninstall CWReader by pressing the

initialization-key once more. However, if you forget to do

this before exiting your contest-program, your system will

hang. But if you set this option to the be the key, with

which you exit from the contest-program, CWReader will be

automatically uninstalled when you are exiting your contest-

program.

Test that a contest-program is loaded (default YES)

If you set this to YES, you are not allowed to initialize

CWReader if you have not loaded a contest-program first.

Usually this may be set to YES, but if you want to enter DOS-

commands with your keyer, this must be set to NO.

Copy spaces (default NO)

This specifies whether you spaces to be copied or not. In a

normal contest-type operation there is no point in copying

the spaces, but when, for example, you are entering DOS-

commands with your keyer, copying spaces is mandatory. When

set to YES, you are asked for space's length in dits (the

delay after which the space is considered a space) and key,

as which the space is entered (almost always as Space).

Trapped key #1-4  (defaults F2: F7, F2 ; Insert: F5, F6, F2 ;

NONE ; NONE)

You can define four trapped keys, which means keys, that are

translated for the contest-program as something else. You

could, for example, make the key 'A' trapped and specify its

sequence as "ABCDE". Then, whenever you press 'A', what would

appear on the screen would be "ABCDE". The trapped keys can

be thought of as simple macros.

Init sequence #1-4   (defaults Alt-V, '2', '2', Enter, Shift-

F2, BkSp * 8 ;

<AFTER599>, Enter, Shift-F6, BkSp * 8 ;

'5', 'N', 'N', Enter, Shift-F7, BkSp * 8 ;

'T', 'U', Space, '5', 'N', 'N', Enter)

These four sequences are sent to the contest-program when

CWReader is initialized. Their purpose is to set the

program's CW-speed to 22 WPM and set CW-memories accordingly.

Between each of the sequences, there is a delay of the length

specified in Global options / Initialization delay.

Entering the sequence is just like entering any other key-

sequence, except that there is an extra-option, Ctrl-Z.

Pressing Ctrl-Z inserts <AFTER599> in the sequence, which

means that the exchange specified in Global options / The

exchange to send after 599 is inserted in place of

<AFTER599>. The maximum length of each init-sequence is 15

with <AFTER599> calculated as set in Global options.

<CONTEST PROGRAM SPECIFIC 'GLOBAL OPTIONS'>

Selecting this options brings up a screen which is just like

the global options -screen. By default all the options are

set to "DEFAULT", which means that their values are taken

from the actual global options.

However, you can change any of the options and it will take

precedence over the corresponding actual global option.

Pressing space returns the options back to DEFAULT.

TIPS FOR CWREADER USERS

This section provides some miscellaneous information/tips about the

operation of CWReader. It's recommended reading for somebody who

has learned to use CWReader but is still a little uncomfortable

with it.

Character spacings

You must be very careful not to send the characters too

close-spaced. For example, in "/P" the space between / and P

is often omited and the call is still well understood, but

CWReader can't say whether -..-..--. is intended to be "/P",

"DDG" or perhaps "NETIME" (hi). These too long 'characters'

are simply skipped by CWReader.

The dits are said to be one unit long and the dahs are three

such units. Character spaces should also be a MINIMUM of

three units! Often, when sending callsigns 'in a hurry' (that

is: in a contest), the spaces just get shortened to two

units; there is no problem when receiving such characters by

ear. But CWReader may have problems reading such CW and it

sure is annoying when you send a call correctly but then

notice it appearing wrong on the screen. (and even more

annoying if you don't!)

Some keyers have (at least my CMOS Super Keyer II has) a

feature called "auto-spacing", which helps to prevent this

kind of errors from happening. It automatically extends any

space longer than two units to at least three units. It's

really a good idea to set your keyer's auto-spacing on, if it

has this feature! The effect is hardly noticed by the

operator when sending with higher speeds (>25 wpm).

An idea of using the external-disable feature

You may have wondered what you could use the external-disable

feature for. Well, one use is to connect a simple switch to

it to make it possible to disable CWReader otherway than

using the Alt-D. For some users flipping a switch is easier

than pushing two keys on the keyboard.

THE DISTRIBUTION POLICY AND REGISTERATION

CWReader is distributed as a shareware-program. You are encouraged

to copy it to your friends, upload it to Bulletin Board Systems

(BBS's, possibly ones that are specialized to HAM-software) and to

spread it all over the globe by any means.

You may try CWReader completely free for some time. You may try it

in a contest, on a DX-pedition or just play with it, giving

commands like "DIR/P" with your paddle.

After you have been using CWReader for some time and have got used

to it and possible even couldn't manage further without it, you

should register your copy. You are NOT asked here to register

"after four weeks from the date when you first received CWReader".

You may decide yourself when you have evaluated CWReader for a long

enough time and when to register. You know, there are no major

contests every weekend and contests are really good opportunities

to test CWReader.

All in all, if you continue using CWReader, you should register.

But if you just try it in a few contests, play with it around a

little and then think "nice program, but I don't need it", you

don't have to register.

Registering CWReader

When the time has come to register your copy of CWReader (see the

above paragraph), you should send a letter, along with $8, to the

following address:

Mikko Noromaa OH3LIM

Kanavam�ki 13 E 23

00840 Helsinki

FINLAND

Include in the letter your name, callsign (if applicable) and

postal address. Also all comments, bug-reports and enhancement-

ideas are welcome. If you want to get the newest version of

CWReader from me in return-mail, include a 5� or 3� diskette or

additional $2 of money. If there is no new version available, your

diskette (or $2) is put on hold and when the next release comes

available, it'll be sent to you.

If you think that $8 is too little to pay for such a brilliant

program as CWReader is, your increased contribution is more than

welcome! And if you can't afford $10, less will do as well. Anyway,

a registeration along with only a few dollars is MUCH better than

no registeration at all!

When you register, you'll get the following benefits:

-  A complete and working CWReader-program (well, that's what

your current version is - the shareware-version is not

crippled in any way)

-  The next version will be sent to you when it becomes

available (if a diskette or $2 sent)

-  When there is a very major new release of CWReader or a very

significant bug-fix, all registered users will get the new

version, be a diskette sent or not

-  Support for CWReader. All questions of registered users will

be answered (preferable sent by normal mail or electronic

mail) and probably those of unregistered users too!

That's not too many benefits - but if I would have offered to send

a registered version with your name on it after receiving your

registeration, send regular updates to all users and so on, I would

have had to at least double the price! I think it's best for all to

have this system with a low price and little benefits.

If you have anything to ask me about CWReader, write to the address

given above. You may also send mail to my internet-address:

Mikko.Noromaa@hut.fi

Hope to see you in contests and pileups! And hope you'll enjoy

CWReader and register it soon!

73 de Mikko Noromaa OH3LIM