CHAT server setup details (Version 4 BPQ)

-----------------------------------------

Getting started

---------------

To run the chat server you must have already installed the G8BPQ

version 4 node software.

Next set up a file which at the moment contains a one line logon

message. The file is

CHAT.CNF

Configure a window in DV which must be at least 22K in size. The

program requires at least 3 parameter values.

The first is the starting stream number  (0<number<64)

The second is the number of streams you wish the chat node to

manage (0<number<10)

The third is the BPQ APPLFLAG number

The fourth is the APPLFLAG number for the *linknode command if this

feature is required(Not yet implemented)

The maximum number is 10. When you open the DV window all the streams

will be disconnected and the messages written to the screen.

SYSOP commands

--------------

From the local terminal you can see all the messages being sent

through the server and also talk to any or all of the users.

When you type it will be displayed in the bottom area of the

sysop screen. Any line entered here which is not preceded by a

/     will be sent out to all users as a message labelled

[SYSOP] xxxxxxxx

If you wish to communicate with one particular user type

/n xxxxxx     where n is the stream number to which the

user is connected. The message will appear on the users screen as

<*SYSOP*> xxxxxxxx

The command  /c  will sent a polite message to all users then

disconnect them before closing the DV window.

The command  /q  will close it all down quickly.

User Commands

=============

The user commands are explained by the help list which is sent in

response to a   /H   command from the user.

Future Enhancements

===================

I have started to implement netwroking of the chat servers. The

technique I intend to use is to allocate a second (hidden) application

name for the server which will be called by a neighbouring server.

When a server has a user connected it will connect to the adjacent

servers and send the messages to it for distribution to the users

connected. I have completed sorted all the problems yet but I am

working on them.