TN211_13.DOC

SYSOP REMOTE PARAMETERS (6-11)

**********************************

Parameter 6, NODES BROADCAST INTERVAL

-------------------------------------

The  NODES  broadcast  interval  controls the  "heart"  of TheNet's dynamic

routing process.   It is this  timer that sets the frequency of broadcasts that

includes node aliases,  callsigns,  routing, and obsolescence information.  The

broadcast timer also initiates action to systematically decrement  failed nodes

from the routing tables.    For proper synchronization of network actions, it's

recommended  the value selected  for the broadcast  interval be the same on all

nodes  in the system.     Short intervals  will  insure greater network routing

reliability but at the expense of increased network overhead.  Longer intervals

decrease overhead but yields poorer routing reliability.    The suggested value

of  1800 seconds  (30 minutes)  achieves a reasonable  balance.     A 30 minute

interval  will  cause  failed nodes to be decremented out of  neighbor  routing

tables  within  3 hours or less.   A setting of "0" will disable the timer (not

recommended).

(Range: 0-65535)

Parameter 7, FRACK

------------------

Specifies  the time in seconds a point-to-point level 2 frame retry will be

sent to a node  or a user if a previous  frame isn't acknowledged.    The value

chosen will  vary with the type of service the node performs.  Small values may

unnecessarily increase the node key-up cycle in multi-user environments.  Large

values on single transmitter links will slow throughput.   Values suggested for

multi-user service are "3-6" and, "1-2" for single/dual transmitter links.

(Range: 1-15)

Parameter 8, MAXFRAME

---------------------

Specifies  the maximum  number of point-to-point level 2 frames that can be

sent to a node or a user without acknowledgement.    Small values on busy high-

speed links  will dramatically increase the node key-up cycle due to  increased

frame  acknowledgement  requirements.   Larger values on solid links will allow

improved  throughput  efficiency.   If keyboard operators are being prematurely

disconnected  on access nodes,  perhaps over-aggressive server TNC parms are at

fault.  Suggested value is 7 frames.

(Range:  1-7)

Parameter 9, LINK MAXIMUM TRIES

-------------------------------

Sets  the number of attempts a point-to-point level 2 circuit will be tried

between a user or a node before a failure is reported.   When level 4 transport

end-to-end  circuits are involved,  the number of times the node transmitter is

keyed  on  a failed link is the product of the values for  parameters 9 and 19.

Suggested value is 7 tries.

(Range:  0-127)

Parameter 10, VALIDATE CALLSIGNS

---------------------------------

Determines  if incoming callsigns  will be  verified  against  the internal

callsign  validation  routine.     It's to  both user and NodeOp's advantage to

implement  callsign  validation.    If  not enabled, a distant user mistyping a

node alias, may have to wait a considerable  length of time before receiving an

appropriate node  response.   If the NodeOp is desiring to allow user downlinks

to  KA-node  aliases,  possibly these  aliases could be selected to satisfy the

callsign  verification  criteria, i.e.,  no  more  than  6 characters with  one

number  included.     For example: CDXB0X (note the zero in  "B0X").   Callsign

validation  ENABLED  also prevents those who forget to  install  their callsign

into  the  TNC,  from  uplinking  to the node.    The node response to a NOCALL

is "Node busy."  Suggested default is "1."

(Range:  0-1)

Parameter 11, HOST MODE CONNECTS

--------------------------------

The primary function of host mode is to allow the NodeOp to service the node

while  on-site.    However,  a user can  connect to a  terminal or other device

attached to the node's RS-232 port if the following conditions are met:

1.  Parameter 11, host mode connects is ENABLED (1).

2.  The terminal or other RS-232 device isn't actively CONNECTED to the node.

Host mode is a single user port, thus allows only one user at a time to CONNECT

to it.   OFF (0), ON (1).

(Range:  0-1)