*** This file is not finished yet... it's very incomplete...

*** First: A message i've written about the speech code:

---------- Cut here -----------------------------------------

If you're *that* keen to use half working code, point your ftp clients

to ftp.pspt.fi/ftp.oh7rba.ampr.org, /pub/oh7lzb/clu/.alpha/speech !

There you can find the speech kit .voc files (zipped).

The LPT port dac code seems to work (it requires a 10 kHz oscillator

signal in the strobe line of the LPT port, plus some sort of D/A

converter in the data lines, you can build one from resistors), and

so does the SoundBlaster code. PTT is on one of the LPT port lines, i

don't remember which one.

The .zip files contain the audio data and index files. You have

a choice of OM and YL voices, English and Finnish (if you want to get

really exotic 8-). Just unzip one of the .zip's to a subdirectory named

clu\speech\.

A few gotchas: do *not* try to do any sort of dos shell while it's

speaking (dos, run, alt-f9, pg...). It will get stuck. It will also most

probably get stuck if it gets a run-time error while speaking. Or anything

like that. It doesn't remove the interrupt handlers or stop the dma

transfers at such situations, and the system will easily get REALLY

surprised with it's pants in it's knees (literally).

It's half done, half working, and very buggy. You've been warned!

It has LOTS of known bugs, which i haven't taken a look at yet. Please

don't send any sort of bug reports for systems which have speech enabled,

until i've cleaned the code up and asked for reports !

---------- Cut here -----------------------------------------

*** And now to the half-written documentation:

����������������������������������������������������������

������������������������͸

�    C  L  U  S  S  E    �

������������������������;

Speech interface

The Free, Speaking DX Cluster / Conference System

����������������������������������������������������������

Written by Heikki Hannikainen, OH7LZB

����������������������������������������������������������

��� Index ���������������������������������������������������������������

0. Disclaimer and a licence to use

1. Introduction

2. Installation

2.1 Hardware

2.2 Software

Appendices:

A: Thank You's

B: Keeping the lawyers happy

��� 0. Disclaimer and a licence to use ����������������������������������

CLUSSE IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND. THE

ENTIRE RISK OF USING THIS PRODUCT IS ASSUMED BY YOU. IN NO

EVENT WILL THE AUTHOR BE LIABLE FOR ANY DAMAGES WHATSOEVER

ARISING OUT OF YOUR USE OR INABILITY TO USE THE SOFTWARE. BY

USING THIS PRODUCT, YOU AGREE TO THE ABOVE LIMITATIONS.

I hereby grant any licenced amateur radio operator to use this

software free of charge. You can modify the program as much as

you like, as long as you don't remove my signatures or distribute

the modified versions without my written permission. This software

may not be used in CB (Citizens Band, 27 MHz) or commercial

environment without my written permission.

Clusse is not for sale. You may not charge ANY money for

a copy of Clusse, except for the price of the media used

for transferring (the disk or the phone bill). Clusse may

be bundled on a CD-ROM or any other such commercial

package, for the price of sending me a free copy of the

product it is bundled with (1 copy per each different product).

If you think you could utilize Clusse in a commercial environment

and you are ready to pay for it, contact the author.

��� 1. Introduction �����������������������������������������������������

In addition to the usual packet radio based DX cluster interface,

Clusse can spell out the DX spots using digitized speech. When the

DX cluster node is equipped with a cheap Sound Blaster compatible

sound card or a simple, home-built LPT port D/A converter, and a

VHF/UHF FM transmitter, DX information can be enjoyed by those

who either don't want or can't install packet radio equipment,

can't carry them everywhere. Not to forget those of us who can't

see the text on the terminal!

Even the most enthusiastic DX'er can leave home during the

DX-pedition of the year - the DX spots will reach him via

his hand-held FM rig.

Even if not connected to a transmitter, the spoken DX

information can be of use at the DX cluster node's location:

just connect it to an amplifier and a loudspeaker, and

you don't need to leave the TV to see what was the DX spot that

made the node's console beep.

The idea and the original implementation of a speaking DX

cluster node came from Sakari Nylund, OH1KH. In 1994 he wrote a

program which took the DX spots from a PacketCluster node and

spoke them. Since his program was not compatible with Clusse, he

suggested that I would implement the audio interface directly in

Clusse. He also provided me with the source code of his original

program and the sound files. Thank You, Sakari.

��� 2. Requirements �����������������������������������������������������

- 300-400 KB of XMS memory to store the speech data.

- Either a Sound Blaster compatible 8-bit sound card (go for the

clones, they're really cheap) OR a home-built LPT port D/A

converter.

- A free LPT (printer) port on the computer for keying the

transmitter and feeding the LPT port D/A converter, if you're

using one.

- If you're using the LPT port DAC, you might need a reasonably

fast computer (feeding data to the DAC is quite time consuming).

A 386 should be enough.

��� 3. Installation �����������������������������������������������������

3.1 Hardware

3.2 Software

Get the Clusse Speech Kit (spe????.zip, where ???? is the

version number) and exctract the files to a directory speech\

under the Clusse root directory (\clu\speech\ in my installation).

The Speech Kit includes the digitized speech libraries. They're

distributed separately because of their size.

There are four versions of the libraries: om_eng.zip and

yl_eng.zip are in English, spoken by male (om) and female (yl)

voice. om_fin.zip and yl_fin.zip are the Finnish versions. The

female voice belongs to Nina, OH1KYE and the male voice to Timo,

OH1NOA.

Each library consists of two files: all.voc, which includes the

actual data, and all.idx, which is an index to the samples in the

library. The speech data is loaded into XMS (yes, you need 300-400

kb of XMS to use the speech kit!), since reading the samples

from disk would usually be too slow to provide smooth speech.

��� Appendices: A - Thank You's �����������������������������������������

I'd like to thank the following people for their precious work:

Female voice:      Nina Alanko, OH1KYE

Male voice:        Timo Klimoff, OH1NOA

Recording:         Raimo M�kel�, Satakunnan Radio, YLE

Digital mastering, original idea and implementation,

original LPT port DAC code: Sakari Nylund, OH1KH

��� Appendices: B - Keeping the lawyers happy ���������������������������

- MS-DOS, Microsoft Windows and Windows 95 are registered trademarks of

Microsoft Corporation.

- DesqView and QEMM are registered trademarks of Quarterdeck Office

Systems.

- OS/2 is a registered trademark of International Business Machines, Inc.

- PacketCluster is a trademark of Pavillion Software, a division of

Harvard Radio.

- PKUNZIP is a trademark of PKWARE, Inc.

- Any trademarks mentioned in this file but not included here by

accident are also acknowledged....