The following programs are provided.

BPQCODE.EXE

The TSR version of the packet switch. It provides a NET/ROM compatible

packet switch, and an interface to application-level software (such as

a BBS).

SWITCH.EXE

This is a foreground version of BPQCODE. It is intended to run only as

a packet switch, and not to support an application system (eg BBS). A

simple terminal program (equivalent to PAC4) is built in, as is the

SYSOP handler from the Data Engine version of the switch. The diffences

from BPQCODE are listed below:

BPQCFG.TXT

This can be the same as for the normal version. The code will assume

NODE=1 and BBS=0, whatever is entered.

Terminal support.

See PAC4 below for details. Ctrl/Y shuts down the whole switch.

IPGATE.

SWITCH.EXE includes IPGATE. See IPGATE.DOC for details.

SYSOP Commands.

The SYSOP feature is similar to SYSOPH, but the password protection

works rather differently. To enter SYSOP mode, enter PASSWORD. If

you are using the console, the switch will respond Ok, and enter

SYSOP mode. If you are accessing the system remotely, it will respond

with 5 random numbers. You must reenter PASSWORD, this time followed

by the corresponding characters from the password string contained

in the file PASSWORD.BPQ. If these are correct the switch responds

OK, otherwise it ignores the message.

So, if the password string is ABCDEFGHIJKLM the following is a

typical sequence:

PASSWORD

BPQ:G8BPQ} 04 02 05 06 03

PASSWORD DBEFC

BPQ:GBBPQ} Ok

Note that the password is converted to upper case before testing, so

PASSWORD.BPQ should contain upper case letters or digits. It should

not contain spaces or other special characters.

There are a number of extra SYSOP commands. These were added primarily

for the Data Engine version, where experimenting with parameters is

rather more difficult, but I've left them in this version as well.

Command		Corresponding BPQCFG parameter

OBSINIT			OBSINIT

OBSMIN			OBSMIN

NODESINT		NODESINTERVAL

L3TTL			L3TIMETOLIVE

L4RETRIES		L4RETRIES

L4TIMEOUT		L4TIMEOUT

T3			T3

IDLETIME		IDLETIME

LINKEDFLAG		ENABLE_LINKED

IDINTERVAL		IDINTERVAL

MINQUAL			MINQUAL

FULLCTEXT		FULL_CTEXT

HIDENODES		HIDENODES

L4DELAY			L4DELAY

L4WINDOW		L4WINDOW

BTINTERVAL		BTINTERVAL

There is also one commend which isnt in the DE version - SAVENODES.

IPGATE.EXE

An IP router to run above the switch (Normally in a DesqView window). See

IPGATE.DOC for further information.

ODIDRV.EXE

An EXTERNAL hardware driver to link switches via Ethernet (using Novell's

ODI interface). See DRIVERS.DOC for further information.

BPQCFG.EXE

This program converts the configuration file BPQCFG.TXT to a binary file

(BPQCFG.BIN) for use by the switch software. It must be run after any

change to BPQCFG.TXT.

BPQNODES.COM

This program dump the ROUTES and NODES tables to a file (BPQNODES). This

file is then used to initialise the above tables when the switch is

reloaded. It should be run before reloading the switch, and if possible

at regular intervals, so that if the system crashes, it comes back with

a reasonably up-to-date view of the network. Many BBS systems allow you

to run a program as part of the forwarding cycle, or at specified times.

It is possible to edit the BPQNODES file. This provides a way of defining

Route Qualities, etc. without changing BPQCFG.TXT, or using SYSOPH.

The file format is: ( {} indicates optional entries - don't include the{})

ROUTE ADD CALL P QUAL {!} {VIA DIGI1 {DIGI2}}

where P is the port, QUAL the Quality, and the optional ! indicates a

locked route. Up to 2 DIGIS may be specified.

NODE ADD ALIAS:CALL CALL-1 P QUAL {!} {CALL-2 P QUAL {!} } {CALL-3 ....}

where CALL-1, CALL-2, CALL-3 are up to 3 alternate routs, which must be

defined (by ROUTES config entries or ROUTE ADD) before being used.

Although locked nodes are allowed, I strongly recommend that they are used

only in exceptional circumstances (eg when using a digi'ed route). Note

also that a locked route to a node will be removed if the system finds 3

higher quality routes to it.

Lines beginning with ; are treated as comments, and fields should be

separated by one (and only one) space.

BPQDUMP.COM

BPQDUMP.COM  is a simple program to dump my code to a file to aid

diagnosis of fault conditions. It creates a file in the current directory

called BPQDUMP (about 98k).

If run without a parameter, it dumps immediately.

If a param is specified, it will loop until the free buffer count drops

below 20 ( the point when the code starts to shut down), then dumps and

exits. Note this mode must only be run in a desqview window.

eg

BPQDUMP		- dumps now

BPQDUMP X	- dumps when buffers < 21

PAC2.COM

PAC2 is a small, simple terminal program, primarily for use with my switch,

but it also works with a normal TNC + COMBIOS. It has one paramter (port

number), so enter PAC2 N, where N is the (virtual) COMBIOS port you want

to access. The program provides a buffered input line, which allows use of

the editing keys ins, del, cul, cur, etc, It allows logging to disk (CTRL/L

toggles this function on and off), and maintains a simple 'Response Time

Monitor' function - useful for seeing how badly the network is performing!

To exit from PAC2, enter CTRL/Y

PAC4.EXE

PAC4 is a simple terminal program, which uses the BPQ Host Mode to talk

to the switch. The program provides a buffered input line, which allows

use of the editing keys ins, del, cul, cur, etc. It allows logging to disk

(CTRL/L toggles this function on and off), and maintains a simple 'Response

Time Monitor' function. A scroll-back buffer is also provided - use PageUp

and PageDown to scroll.

The program has one required paramter (port number), and a second optional

one (M for mono mode). So possible command lines are:

PAC4 63			; to use stream 63

PAC4 1 M		; to use stream 1, if you have a mono monitor.

Connection is made to the switch when a message is input. The ESC key

disconnects you from the switch.

Monitoring is toggled on/off using F1. F2 toggles MCOM mode (monitoring of

control frames as well as info frames). F3 allows you to set MMASK. Press

F3, then key up to 4 hex digits, terminated with a newline. Each bit of

MMASK enables the monitoring of the corresponding radio port. Bit 0

enables port 1, bit 15 port 16. Any combination of bits may be set. The

current monitor settings are displayed on the status line.

To exit from PAC4, enter CTRL/Y

SYSOPH.COM

This program allows the manipulation of certain parameters within the

switch. The program may be run as a normal DOS program, either in a

Desqview window or standalone, or it may run as a TSR. The TSR mode would

normally be used if you were running a BBS system that doesnt need

DesqView (eg AA4RE). In TSR mode, commands cannot be entered locally, and

the SAVENODES command is disabled. It either mode, it may be accessed over

the network.

The system has now been modifed to use BPQHOST mode rather than a virtual

TNC2 port. Program SYSOP.COM is withdrawn.

Installation.

1. Add a suitible application name as the third APPLICATIONS Param, ie

APPLICATIONS=BBS,,*SYSOP

I suggest you use a name starting with a '*', as this will 'hide' the

command from the Valid Commands display.

2� Creat� � file� PASSWORD.BPQ, containing a string of up to 80 characters.

3. Run program SYSOPH, with a single parameter of the stream to be used,

either on its own, or in a DesqView window. If a second parameter of

'RESIDENT' is specified, the program will become a TSR routine.

Commands may now be entered directly on the PC, (unless in TSR mode) or

remotely by connecting to the Node, and entering the APPL name (eg *SYSOP).

When accessed remotely, a password prompt is sent, containing 5 random

numbers in the range 1 to the length of the PASSWORD.BPQ string. The first

command entered must be the corresponding five letters from the password

string, or the user will be disconnected.

Available Commands.

The following commands are supported:

BYE - Disconnect remote user (even if entered from console)

TXDELAY

MAXFRAME

FRACK

RESPTIME

PACLEN

RETRIES

QUALITY

PERSIST

TXTAIL

XMITOFF

DIGIFLAG

DIGIPORT

USERS

VALIDCALL

L3ONLY

BBSALIAS

SOFTDCD

The above display or update the parameter for a specified port,

eg   PACLEN 1 100

PERSIST 2 50

If the third parameter is omitted, the current value is displayed.

Setting XMITOFF will supress all TX activity on a port (including CWID,

beacons, etc). I put it in so I could monitor the satellites without

causing QRM, but it could be used as a way of shutting down a port in

an emergency.

Setting BBSALIAS will cause connects to the port call or port alias to

go to the BBS rather than the node.

It is not currently possible to update VALIDCALLS, but you can display

the current setting.

*** Warning ***

At the moment the FRACK value is in 1/3sec units, and the TXDELAY is in

50ms units for HDLC cards, and 10ms units for KISS TNCs. I'll be sorting

this out when I get the time to rationalize the timer code, but in the

meantime, I suggest you make a note of the current values before you

start fiddling.

Also KISS users should note that the params are only sent to the TNC every

5 mins or so, so the updated values may not take effect for a while.

ROUTES

To display ROUTES table, enter ROUTES

To display entry for a particular neighbour, enter ROUTES Call Port,

eg   ROUTES G8BPQ-2 2

To update an entry, enter ROUTES Call Port Params

eg   ROUTES G8BPQ-2 2 100	- Set quality to 100

ROUTES G8BPQ-2 2 !         - Toggle 'Locked Route' flag

To clear the stats counters on a route, enter:

ROUTES G8BPQ-2 2 Z

SAVENODES (Not available in TSR mode).

Writes the ROUTES and NODES tables to the BPQNODES file.

The SAVENODES function may also be invoked directly from the DOS command

line - this function replaces BPQNODES.COM

ie.  SYSOP SAVENODES

John Wiseman, G8BPQ

31.8.92

#